Assignment 1 - Phase 2: Pond Catchment Analysis Backend Report

| Author / Student: | Madhu |
| --- | --- |
| Instructor: | Shivam Kushwaha |
| Submission Date: | September 2, 2026 |
| Course Assignment: | Assignment 1 - Phase 2: Pond Catchment Analysis Backend |
| GitHub Repository: | https://github.com/madhu328/pond-catchment-api |

1. Project & API Route Overview

| GitHub Repository | https://github.com/madhu328/pond-catchment-api |
| --- | --- |
| Primary API Endpoint (Network / TA) | http://10.50.33.238:3313/analyzeContour |
| Local Machine Endpoint | http://localhost:3313/analyzeContour |
| Public WAN Endpoint | http://103.147.138.252:3313/analyzeContour |
| Alias API Endpoint | http://10.50.33.238:3313/findCatchment |
| Required Form Field | contour_map (File upload: .kml or .kmz) |
| HTTP Method | POST |
| Interactive Docs (Swagger UI) | http://10.50.33.238:3313/docs (or http://localhost:3313/docs) |
| Interactive Upload Interface | http://10.50.33.238:3313/ (or http://localhost:3313/) |

2. Catchment Estimation Approach & Methodology

The backend implements a fully generalized hydrological & terrain analysis pipeline that dynamically parses coordinate and elevation data from KML/KMZ files without relying on hardcoded coordinates or bounding boxes.

+-------------------------------------------------------+ ¦ Uploaded KML/KMZ File ('contour_map' form field) ¦ +-------------------------------------------------------+ ¦ v +-------------------------------------------------------+ ¦ 1. Robust KML/KMZ Parser (XML / zipfile parsing) ¦ ¦ - Extracts 3D coordinates (Lon, Lat, Elevation) ¦ +-------------------------------------------------------+ ¦ v +-------------------------------------------------------+ ¦ 2. DEM Builder (SciPy Grid Interpolation) ¦ ¦ - Projects points & generates elevation grid matrix¦ +-------------------------------------------------------+ ¦ v +-------------------------------------------------------+ ¦ 3. D8 Flow Direction & Flow Accumulation Algorithm ¦ ¦ - Calculates steepest slope & upstream drainage ¦ +-------------------------------------------------------+ ¦ v +-------------------------------------------------------+ ¦ 4. Pond Site Selection & Catchment Delineation ¦ ¦ - Selects natural convergence point ¦ ¦ - Backtracks contributing upstream area ¦ +-------------------------------------------------------+ ¦ v +-------------------------------------------------------+ ¦ 5. Structured JSON Output Generation ¦ ¦ - Lon/Lat, Elevation, Area (m² & Ha), Boundary ¦ +-------------------------------------------------------+

Key Technical Steps

1. Dynamic KML/KMZ Parsing (app/kml_parser.py): Parses XML placemarks, LineStrings, and Polygons. Automatically unzips .kmz files to locate doc.kml. Extracts all 3D tuples (Longitude, Latitude, Elevation in meters).

2. Digital Elevation Model (DEM) Interpolation (app/dem_builder.py): Converts geographic coordinates into an equidistant local meter-based grid. Uses SciPy griddata (Cubic & Nearest-Neighbor interpolation) to build a continuous regular raster grid of elevation values.

3. D8 Flow Routing & Accumulation (app/catchment.py): Computes the direction of steepest descent from each cell to its 8 neighbors (D8 Flow Direction). Resolves topological sorting to calculate Flow Accumulation—the exact number of upstream grid cells draining into each individual cell.

4. Optimal Pond Placement (app/catchment.py): Identifies cells combining high flow accumulation (high runoff collection potential) with low relative elevation. Selects the optimal cell for village pond construction.

5. Catchment Delineation & Area Calculation: Performs a recursive reverse traversal from the target pond cell to identify all contributing cells. Calculates exact catchment area: Area (m²) = Contributing Cells × (Grid Resolution)².

3. Demonstration on Provided Sample Map (contours_1m.kml)

Execution Details

• Input File: contours_1m.kml (6.71 MB)
• Processing Time: ~2.16 seconds
• Grid Resolution: 10.0 meters/cell
• Grid Dimensions: 264 × 324 grid cells

Returned Results Summary

Recommended Pond Site Location:
  - Latitude: 21.246056° N
  - Longitude: 81.289336° E
  - Site Elevation: 268.0 meters

Catchment Hydrology:
  - Total Catchment Area: 35,700.0 m² (3.57 Hectares)
  - Contributing Cells: 357 cells
  - Minimum Elevation: 268.0 m
  - Maximum Elevation: 284.88 m

4. API Specification & Documentation

cURL Command Example

curl -X POST "http://10.50.33.238:3313/analyzeContour" \ -F "contour_map=@contours_1m.kml"

Postman Setup Instructions

1. Set HTTP Method to POST.
2. Enter URL: http://10.50.33.238:3313/analyzeContour.
3. Under the Body tab, select form-data.
4. Set Key = contour_map (change type dropdown from Text to File).
5. Attach contours_1m.kml.
6. Click Send

Sample Output Response (JSON)

{ "input_file": "contours_1m.kml", "processing_time_seconds": 2.16, "dem_resolution_meters": 10.0, "grid_size": { "rows": 264, "cols": 324 }, "recommended_pond_location": { "longitude": 81.28933599489764, "latitude": 21.24605558428102, "elevation_m": 268.0 }, "catchment": { "area_m2": 35700.0, "area_hectares": 3.57, "num_contributing_cells": 357, "elevation_min_m": 268.0, "elevation_max_m": 284.88, "boundary_polygon_lonlat": [ [81.28846546444733, 21.24542323664919], [81.28807856202498, 21.245603907401144], [81.28788511081379, 21.24569424277712], [81.2877883852082, 21.245784578153092], [81.28759493399703, 21.245965248905044], [81.28749820839144, 21.246416925784924], [81.28769165960261, 21.246958938040777], [81.28807856202498, 21.247410614920657], [81.28827201323615, 21.247591285672605], [81.2887556412641, 21.247591285672605], [81.28894909247528, 21.247410614920657], [81.28933599489764, 21.247049273416753], [81.2896261717144, 21.24668793191285], [81.28933599489764, 21.24605558428102], [81.28856219005291, 21.24542323664919], [81.28846546444733, 21.24542323664919] ] }, "note": "Runoff volume and rainfall-based storage estimates will be integrated in the next phase using a rainfall API, using this catchment area as the input." }

5. Code Extensibility & Future Phase Readiness

1. Modular Architecture:
   • app/kml_parser.py: Standalone parser supporting any standard KML/KMZ containing point, linestring, or polygon contour elements.
   • app/dem_builder.py: Independent DEM generator scalable to custom grid resolutions.
   • app/catchment.py: Pure mathematical D8 hydrology engine.
   • app/main.py: Clean RESTful API wrapper using FastAPI.

2. Phase 3 Integration Readiness:
   • The calculated catchment area (35,700.0 m² / 3.57 ha) directly serves as the input parameter for precipitation runoff equations (Q = C · I · A).
   • Future rainfall APIs (Open-Meteo / IMD) can easily query historical or real-time precipitation data (I) using the recommended_pond_location coordinates (21.246056° N, 81.289336° E).